| UNIT 1 |  |
| --- | --- |
| UNITS AND DIMENSIONS |  |
| 1 | Convert 100 °C to K, °F and °R |
| 2 | Convert 340 mm Hg pressure to inch of water and kilo Pascal |
| 3 | Find the equivalent of 35 psi in inches of mercury |
| 4 | The thermal conductivity of aluminium is 117 Btu/ hr ft °F. Find the value of thermal conductivity in w/ m°C |
| 5 | The value of heat transfer coefficient is given as 396 Btu/ hrft2° F. what is its value in kcal/m2hr°C |
| 6 | Find the Btu equivalent of kWh |
| 7 | The mass velocity of a fluid is 5 kg/m2s. Convert this to lb/ft2min 1kg=1000gm 1lb=454 gm 1ft= 0.3048 m 1min= 60 s |
| 8 | The flow rate of water through a pipe is reported as 15 cubic feet per minute. Taking density of water as 1 g/cm3, calculated the mass flow rate in kg/s |
| 9 | Express kgf/cm2 as N/m2 1 kgf = 9.8 N |
| 10 | The weight of an object is 300N at a location where acceleration due to gravity is 9.81 m/s2. - Determine the mass of the object in kilograms - Express the mass in the FPS system |
| 11 | The value of heat transfer coefficient in a particular heat exchange operation is found to be 200 Btu/h ft2 F. Convert its value in W/m2 °C. |
| 12 | Iron metal weighs 500 lb and occupies a volume of 29.25 litres. Find the density in kg/m3 |
| CONVERSION OF EQUATIONS |  |
| 1 | The heat transfer coefficient is given by the correlation h= α CpG0.8/ D0.2 =16.6 CpG0.8/ D0.2 where Cp- the heat capacity, Btu/ lb°F, D- internal diameter of pipe, inch G mass velocity, lb/ft2 s , h- heat transfer coefficient Btu/ hr ft2 °F If the equation is written in the new unit system as h’= α’ Cp’G’0.8/ D’0.2 =16.6 Cp’G’0.8/ D’0.2 Where Cp ’ is the heat capacity, kcal/ kg °C D- internal diameter of pipe, cm G -mass velocity, kg/m2 s h -heat transfer coefficient kcal/ hr m2° C, find the value of the constant 1Btu=252 cal = 0.252 kcal 1ft=0.3048 m 1inch= 2.54 cm 1lb=454 gm = 0.454 kg 1°C=1.8°F |
| 2 | The pressure drop for a fluid flowing through a packed bed of solids is given by the relation: ∆P = 3.61µ0.15Hρ0.85ν1.85Dp1.15 Where ∆P is pressure drop in Pa, µ is fluid viscosity in poisegcm s, H is the bed depth in metres, 𝞺 is fluid density in kg/m3, 𝝼 is fluid velocity in m/s and Dp is packing diameter in metres. Modify the relation such that the pressure drop is still in Pa, but other terms are in FPS units. |
| 3 | An empirical equation for calculating the heat transfer coefficient is given by the relation 0.023 G0.8 k 0.67 Cp 0.33 h = , Btu/ hr ft2 F D0.2 0.47 h -heat transfer coefficient, Btu/hr ft2 F G -mass velocity of the liquid, lb/ hr ft2 K -thermal conductivity of the liquid, Btu/hr ft F Cp-heat capacity of the liquid, Btu/lb F -Viscosity of the liquid, lb/ft hr D - diameter of the pipe, ft Convert this empirical equation to SI system. |
| 4 | Rate of heat loss per unit area is given by qA BTUhr ft2 = 0.5 x ΔTF1.25Dft0.25 Convert this relation into kCals m2 |
| 5 | Vapour pressure of benzene in the range of 7.5 °C to 104 °C is given by log10 (p) = 6.9057 – 1211(T + 220.8), , where T is in °C and p is in torr: 1 torr = 133.3 N/m2. Convert it to SI units. |
| 6 | The heat capacity equation for carbon dioxide is as follows Cp=6.339+10.14x10-3 TK - 3.415x10-6TK2 where Cp is the heat capacity, cal/gm mole K and TK is the temperature in degree Kelvin. It is desired to transform the equation in to a new form where CP’ is the heat capacity in Btu/lb° F and temperature TF° is in Fahrenheit scale. |
|  | BASIC CHEMICAL CALCULATIONS |
| 1 | An aqueous solution contains 40% Na2CO3 by weight. Express the composition in mole%. |
| 2 | An aqueous solution of NaCl is prepared by dissolving 25kg NaCl in 100 kg water. Find the weight % and mole% composition of solution |
| 3 | The analysis of magnetite ore yields 81% MgCO3, 14%SiO2 and 5%H2O by weight. Convert the analysis in to mole% |
| 4 | A solution of sodium chloride in water contains 200 gms of sodium chloride per liter. The density of this solution is 1.1 gm/cc. Calculate a. composition in weight % b. volumetric percentage of water c. composition in mole % d. Molarity e. molality f. normality |
| 5 | A solution of caustic soda in water contains 20% NaOH by weight at 60 C. The density of solution is 1.2 gm/cc. Find the molality, Molarity and normality of the solution. |
| 6 | A sample of sea water contains 35000 PPM solids. Express the concentration of the solids in weight percentage |
| 7 | The nitrogen content in a urea sample is estimated as 45 % by weight. Find the purity of urea sample. |
| 8 | An aqueous solution contains 19% NH3, 65.6%NH4NO3 and 6% urea by weight. Calculate the available nitrogen content of solution. |
| 9 | An aqueous solution contains 19% NH3, 65.6%NH4NO3 and 6% urea by weight. Calculate the available nitrogen content of solution. |
| 10 | A product gas from a reaction has the composition by weight Cl2-67%, Br2-28%, O2-5%. Using ideal gas law, calculate a. the composition of gas by volume b. density of gas mixture in gm/lit at 25°C c. specific gravity of mixture and d. average molecular weight. |
| 11 | The gas acetylene is produced according to the following reaction CaC2+2H2O C2H2+Ca (OH)2 Calculate the number of hours of service that can be derived from 1kg of carbide in an acetylene lamp burning 1.5m3 of gas per hour at a temperature of 30°C and pressure of 743 mm. |
| 12 | A gas mixture is found to contain the following composition by volume. Ethylene 30.6%,Benzene24.5% oxygen 1.3% methane 15.5% ethane 25% nitrogen 3.1%. Find a. the composition in mole% b. composition in weight % c. average molecular weight d. density of the gas at NTP. |
| 13 | The gas from petroleum refinery has the following composition by weight methane-27.13% ethane 11.3% ethylene 26.37% propane 11.61% propylene 12.66% and n-butane 10.93%. Find a. composition in mole % b. density of mixture at 25°Cand 7500 mm Hg c. average molecular weight. |
| 14 | In the manufacture of HNO3, initially NH3 and air are mixed at 7 atmg. and 650 °C. The composition of gas mixture n volume% is as followsN2-70.5%, O2-18.8%, H2O-1.2%,NH3-9.%%.Find the density of gas mixture using ideal gas law. |
| 15 | A chimney gas has the following composition by volume CO2-9.5,CO-0.2%,O2-9.6% and N2-80.7%.Using ideal gas law calculate a. composition of gas by weight% b. volume occupied by 100 kg of gas at 25°C and atm.pressure c. density of the gas mixture in kg/m3 |