IM233AI – WORK SYSTEMS DESIGN

UNIT III

Topic ID 6: Work Measurement and Time Study

| Q No | Question | Marks | Difficulty Level | BT Level | COs |
| --- | --- | --- | --- | --- | --- |
| 1. | Define a "Standard Time" according to Groover and explain the four specific conditions that must be met for a time standard to be valid. | 8 | L | 2 | 3 |
| 2. | Rank the five primary work measurement methods by accuracy and justify the high ranking of Predetermined Motion Time Systems (PMTS). | 8 | M | 2 | 3 |
| 3. | Rank the work measurement methods by "Relative Application Speed" (Time to set the standard) and explain why Historical Data is the fastest. | 8 | M | 2 | 3 |

Topic ID 7: Allowances and Time Study

| 4 | Distinguish between Personal, Fatigue, and Delay (PFD) allowances. Why are they necessary in determining the final standard time? | 8 | M | 2 | 1 |
| --- | --- | --- | --- | --- | --- |
| 5 | Explain the difference between "Constant Fatigue Allowances" and "Variable Fatigue Allowances." Provide examples of factors that influence variable fatigue. | 8 | M | 3 | 1 |
| 6 | What are "Contingency Allowances"? How do they differ from "Policy Allowances"? | 8 | M | 2 | 1 |
|  | A worker's observed time is 2.0 min, and their performance rating is 110%. The company uses a PFD allowance of 15%. Calculate the Normal Time and the Standard Time. | 8 | M | 3 | 1 |
| 7 | Explain the "Internal" vs. "External" work elements in a worker-machine system. How do allowances apply differently to these? | 8 | M | 2 | 1 |
| 8 | Outline the step-by-step procedure for determining the Standard Time of a manual task starting from the initial observation. | 8 | M | 2 | 3 |
| 9 | Why is it necessary to break a task into "Work Elements" before timing it? Provide ANY six rules for defining these elements. | 8 | M | 2 | 3 |
| 10 | An operator takes an average of 1.20 minutes to complete a cycle. The analyst rates the worker at 115%. If one element in the cycle was a machine-controlled process taking 0.40 minutes, calculate the Normal Time for the manual portion and the total cycle. | 8 | M | 3 | 3 |
| 11 | Discuss the procedure for determining standard time in "Machine-Paced" tasks. How do allowances differ for the machine portion vs. the manual portion? | 8 | M | 2 | 3 |
| 12 | The snap back timing method was used to obtain average times for work elements in one work cycle. The times are given below. Element d is a machine-controlled element and the time is constant. Elements a,b,c,e and f are operator controlled and were performance rated at 80%; however elements e and f are performed during the machine controlled element d.The machine allowance is zero and the PFD allowance is 14%. Determine the normal time and standard time for the cycle. Element; a; b; c; d; e; f. Observed Time (min); 0.24; 0.30; 0.17; 0.76; 0.26; 0.14. | 8 | M | 3 | 3 |
| 13 | The continuous timing method in direct time study was used to obtain the elemental times for a worker machine task as indicated.Element c is a machine controlled and the time is constant.Elements a,b ,d,e and f are operator controlled and external to machine cycle and they are performance rated at 80%.If the machine allowance is 25% and worker PFD allowance is 15%, determine the normal time and standard time for the work cycle. Element; a; b; c; d; e; f. Observed Time(min); 0.18; 0.30; 0.88; 1.12; 1.55; 1.80. | 8 | M | 3 | 3 |
| 14 | The continuous timing method in direct time study was used to obtain the elemental times for a worker machine task as indicated. Element c is a machine controlled and the time is constant. Elements a,b ,d,e and f are operator controlled and external to machine cycle and they are performance rated at 90%.If the machine allowance is 12% and worker PFD allowance is 8%, determine the normal time and standard time for the work cycle. Element; a; b; c; d; e; f. Observed Time(min); 0.24; 0.38; 0.88; 1.04; 1.65; 1.90. | 8 | M | 3 | 3 |
| 15 | The continuous timing method in direct time study was used to obtain the elemental times for a worker machine task as indicated. Element c is a machine controlled and the time is constant. Elements a,b ,d,e and f are operator controlled and external to machine cycle and they are performance rated at 80%.If the machine allowance is 25% and worker PFD allowance is 15%, determine the normal time and standard time for the work cycle. Element; a; b; c; d; e; f. Observed Time(min); 0.12; 0.28; 0.78; 1.04; 1.45; 1.80. | 8 | M | 3 | 3 |
| 16 | The continuous timing method in direct time study was used to obtain the elemental times for a worker machine task as indicated. Element c is a machine controlled and the time is constant. Elements a,b ,d,e and f are operator controlled and external to machine cycle and they are performance rated at 90%.If the machine allowance is 35% and worker PFD allowance is 25%, determine the normal time and standard time for the work cycle. Element; a; b; c; d; e; f. Observed Time(min); 0.12; 0.28; 0.78; 1.04; 1.45; 1.80. | 8 | M | 3 | 3 |
| 17 | The continuous timing method in direct time study was used to obtain the elemental times for a worker machine task as indicated. Element c is a machine controlled and the time is constant. Elements a,b ,d,e and f are operator controlled and external to machine cycle and they are performance rated at 90%.If the machine allowance is 35% and worker PFD allowance is 25%, determine the normal time and standard time for the work cycle. Element; a; b; c; d; e; f. Observed Time(min); 0.18; 0.30; 0.88; 1.12; 1.55; 1.80. | 8 | M | 3 | 3 |
| 18 | The flyback timing method was used to obtain average times for work elements in one work cycle. The times are given below. Element d is a machine-controlled element and the time is constant. Elements a,b,c,e and f are operator controlled and were performance rated at 90%; however elements e and f are performed during the machine controlled element d.The machine allowance is zero and the PFD allowance is 14%. Determine the normal time and standard time for the cycle. Element; a; b; c; d; e; f. Observed Time (min); 0.18; 0.24; 0.21; 0.74; 0.26; 0.24. | 8 | M | 3 | 3 |
| 19 | The snap back timing method was used to obtain average times for work elements in one work cycle. The times are given below. Element d is a machine-controlled element and the time is constant. Elements a,b,c,e and f are operator controlled and were performance rated at 90%; however elements e and f are performed during the machine controlled element d.The machine allowance is zero and the PFD allowance is 14%. Determine the normal time and standard time for the cycle. Element; a; b; c; d; e; f. Observed Time (min); 0.24; 0.30; 0.19; 0.96; 0.26; 0.24. | 8 | M | 3 | 3 |

Topic ID 8: PMTS

| 20 | Discuss the advantages of PMTS over direct stopwatch time study as highlighted by Groover. | 8 | L | 2 | 3 |
| --- | --- | --- | --- | --- | --- |
| 21 | Distinguish between first-level (MTM-1) and higher-level (MTM-2, MTM-3) PMT systems. | 8 | M | 2 | 3 |
| 22 | What is MOST (Maynard Operation Sequence Technique)? Explain its focus on "Sequence Models" rather than individual motions. | 8 | M | 2 | 3 |
| 23 | Break down the "General Move" sequence model (ABGABPA). Define each parameter. | 8 | M | 2 | 3 |
| 24 | Compare "General Move" vs. "Controlled Move" in Basic MOST. Give an example for each. | 8 | M | 3 | 3 |
| 25 | Distinguish between Mini-MOST, Basic MOST, and Maxi-MOST based on cycle length and repetitiveness. | 8 | M | 2 | 3 |
| 26 | What is "Admin MOST"? Provide three examples of tasks where it would be applied. | 8 | M | 3 | 3 |
| 27 | Discuss "MOST for Windows" (Computerized MOST). What are the advantages of using software over manual data cards? | 8 | M | 2 | 3 |
| 28 | Outline the systematic procedure for applying a Predetermined Motion Time System (PMTS) to establish a time standard for a manual task. | 8 | L | 2 | 3 |
| 29 | Describe the "Tool Use" sequence model. List four common tool-use actions it covers. | 8 | M | 2 | 3 |